Caledonian Mining Cables

AS/NZS 2802:2000 Reeling & Trailing Cables

Type 412 1.1/1.1KV

» Applications .............................................................................................................

These cables with green/yellow earths and pliable armour for mechanical protection may be used in applications where damage is likely and armour can reduce cases of costly downtime, suitable installed as feeder cables in sand mining operations.

» Standards .................................................................................................................

AS/NZS 2802:2000

AS/NZS 1125

AS/NZS 3808

AS/NZS 5000.1

» Construction ............................................................................................................

Outer Heavy Duty PCP Sheath Inner PCP Sheath

Stranded Tinned Copper Conductor EPR Insulation

Central Filler Insulation Screen

Interstitial Earth Conductor Pliable Galvanized Steel Armour

3×Conductors: Flexible stranded tinned annealed copper conductor.

Insulation: EPR.

Insulation Screen: Semiconductive elastomer.

Filler: Elastomer centre filler.

3×Interstitial Earth Conductor: EPR covered flexible stranded tinned copper conductor.

Inner Sheath: PCP sheath. CPE/CSP sheath can be oﬀered upon request.

Pliable Armour: Galvanized low carbon (mild) steel strands.

Outer Sheath: Heavy duty PCP sheath. Heavy duty CPE/CSP sheath can be oﬀered upon request.

28	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables

AS/NZS 2802:2000 Reeling & Trailing Cables

»  Dimensions and Weight ..........................................................................................

| Nominal Conductor Area | Strand Size | Insulation Thickness | Earth Conductor |  | Pliable Armour Size | Thickness of Sheath |  | Nominal Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Strand Size | Thickness of Covering |  | Inner | Outer |  |  |
| mm² | No/mm | mm | No/mm | mm | No/mm | mm | mm | mm | kg/100m |
| Type 412.1 Class2 |  |  |  |  |  |  |  |  |  |
| 16 | 126/0.40 | 1.6 | 81/0.30 | 0.6 | 7/0.90 | 2.5 | 3.8 | 38.3 | 265 |
| 25 | 209/0.40 | 1.6 | 81/0.30 | 0.6 | 7/0.90 | 2.5 | 3.8 | 38.4 | 294 |
| 35 | 285/0.40 | 1.6 | 81/0.30 | 0.6 | 7/0.90 | 2.5 | 4.0 | 44.6 | 402 |
| 50 | 380/0.40 | 1.7 | 120/0.30 | 0.8 | 7/0.90 | 2.5 | 4.4 | 49.2 | 500 |
| 70 | 203/0.67 | 1.8 | 39/0.67 | 0.8 | 7/0.90 | 2.5 | 4.8 | 55.4 | 655 |
| 95 | 259/0.67 | 2.0 | 48/0.67 | 0.8 | 7/0.90 | 2.5 | 5.4 | 60.1 | 775 |
| 120 | 336/0.67 | 2.1 | 60/0.67 | 1.0 | 7/0.90 | 3.5 | 5.8 | 68.1 | 990 |
| 150 | 427/0.67 | 2.3 | 77/0.67 | 1.0 | 7/0.90 | 3.5 | 6.3 | 73.8 | 1186 |
| 185 | 518/0.67 | 2.5 | 91/0.67 | 1.0 | 7/0.90 | 3.5 | 6.8 | 80.2 | 1360 |
| 240 | 672/0.67 | 2.8 | 119/0.67 | 1.2 | 7/0.90 | 3.5 | 7.5 | 88.7 | 1670 |
| 300 | 854/0.67 | 3.0 | 156/0.67 | 1.2 | 7/1.25 | 4.5 | 8.2 | 100.9 | 2200 |

www.caledonian-cables.co.uk  www.addison-cables.com	29