Caledonian Mining Cables

AS/NZS 1972:2006 Cables

Type 2S 1.1/1.1KV & 3.3/3.3KV Individually Screened

» Applications .............................................................................................................

These individually copper screened cables are used for wiring of machines or between machines and equipment where a rubber cable is desired. These cables are also used for longwall lighting circuits, and may contain pilot and control cores or twisted pair and screened cores.

» Standards .................................................................................................................

AS/NZS 1972:2006

AS/NZS 1125

AS/NZS 3808

» Construction ............................................................................................................

Stranded Tinned Copper Conductor EPR Insulation

Elastomer Centre Filler Individual Composite Screen Pilot/Control Cores

Heavy Duty CPE Sheath

Conductors: Stranded tinned annealed copper conductor.

Insulation: EPR.

Filling: Elastomer centre filler.

Pilot/Control Cores: EPR covered and composite screened flexible stranded tinned copper conductor.

Individual Composite Screen (earth conductor): Tinned annealed copper braiding interwove with polyester yarn.

Sheath: Heavy duty CPE sheath.

46	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables

AS/NZS 1972:2006 Cables

»  Dimensions and Weight ..........................................................................................

| Number of Core | Nominal Conductor Area | Strand Size | Insulation Thickness | Core Screen |  | Pilot Conductor |  |  | Nominal Sheath Thickness | Nominal Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Strand Size | Area of Screen | Number of Pilots | Strand Size | Thickness of Covering |  |  |  |
|  | mm² | No/mm | mm | mm | mm² |  | No/mm | mm | mm | mm | kg/100m |
|  | Type 2S 1.1/1kV Individually Screened |  |  |  |  |  |  |  |  |  |  |
| 3 | 10 | 77/0.40 | 1.2 | 0.2 | 7.9 | 3 | – | – | 1.8 | 22.1 | 69 |
| 4 | 10 | 77/0.40 | 1.2 | 0.2 | 7.9 | 4 | – | – | 1.8 | 23.0 | 93 |
| 2 | 16 | 126/0.40 | 1.2 | 0.2 | 9.0 | 2 | – | – | 1.8 | 22.0 | 81 |
| 3 | 16 | 126/0.40 | 1.2 | 0.2 | 9.0 | 3 | – | – | 1.8 | 23.4 | 100 |
| 4 | 16 | 126/0.40 | 1.2 | 0.2 | 9.0 | 4 | – | – | 1.8 | 26.8 | 135 |
| 3 | 10 | 77/0.40 | 1.2 | 0.2 | 7.9 | 3 | 30/0.20 | 1.0 | 1.8 | 22.5 | 87 |
| 2 | 16 | 126/0.40 | 1.2 | 0.2 | 9.0 | 2 | 30/0.20 | 1.0 | 1.8 | 22.7 | 88 |
| 3 | 16 | 126/0.40 | 1.2 | 0.2 | 9.0 | 3 | 30/0.20 | 1.0 | 1.8 | 27.7 | 130 |
| 3 | 25 | 209/0.40 | 1.4 | 0.2 | 11.3 | 3 | 30/0.20 | 1.0 | 1.8 | 29.2 | 165 |
| 3 | 35 | 285/0.40 | 1.4 | 0.2 | 12.4 | 3 | 30/0.20 | 1.0 | 1.8 | 31.6 | 200 |
| 3 | 50 | 380/0.40 | 1.6 | 0.25 | 17.5 | 3 | 30/0.20 | 1.0 | 1.9 | 36.1 | 260 |
|  | Type 2S 1.1/1kV Individually and Collectively Screened |  |  |  |  |  |  |  |  |  |  |
| 30 | 1.5 | 30/0.25 | 1.0 | 0.3 | 14.0 | – | – | – | 1.9 | 32.6 | 170 |
|  | Type 2S 3.3/3.3Kv Individually Screened |  |  |  |  |  |  |  |  |  |  |
| 3 | 10 | 77/0.40 | 3.0 | 0.2 | 11.3 | 3 | 30/0.25 | 1.0 | 1.8 | 28.9 | 130 |
| 3 | 16 | 126/0.40 | 3.0 | 0.2 | 12.4 | 3 | 30/0.25 | 1.0 | 1.8 | 31.3 | 160 |
| 3 | 25 | 209/0.40 | 3.0 | 0.2 | 13.6 | 3 | 30/0.25 | 1.0 | 1.9 | 34.8 | 210 |
| 3 | 35 | 285/0.40 | 3.0 | 0.2 | 15.3 | 3 | 30/0.25 | 1.0 | 2.0 | 37.8 | 250 |
| 3 | 50 | 380/0.40 | 3.0 | 0.2 | 17.0 | 3 | 30/0.25 | 1.0 | 2.1 | 41.3 | 305 |
| 3 | 70 | 203/0.67 | 3.0 | 0.3 | 30.5 | 3 | 30/0.25 | 1.0 | 2.2 | 46.8 | 415 |
| 3 | 95 | 259/0.67 | 3.0 | 0.3 | 30.5 | 3 | 30/0.25 | 1.0 | 2.3 | 49.6 | 490 |
| 3 | 120 | 336/0.67 | 3.0 | 0.4 | 47.5 | 3 | 30/0.25 | 1.0 | 2.5 | 55.0 | 620 |

www.caledonian-cables.co.uk  www.addison-cables.com	47