Caledonian Mining Cables

AS/NZS 1972:2006 Cables

Type 2S 1.1/1.1KV & 3.3/3.3KV Collectively Screened

» Applications .............................................................................................................

These collectively copper screened cables are used for wiring of machines or between machines and equipment where a rubber cable is desired. These cables are also used for longwall lighting circuits, and may contain pilot and control cores or twisted pair and screened cores.

» Standards .................................................................................................................

AS/NZS 1972:2006

AS/NZS 1125

AS/NZS 3808

» Construction ............................................................................................................

Conductors: Stranded tinned annealed copper conductor.

Insulation: EPR.

Filling: Elastomer centre filler.

Bedding: Polyester tape.

Stranded Tinned Copper Conductor EPR Insulation

Elastomer Centre Filler Polyester Tape Composite Screen Heavy Duty CPE Sheath

Composite Screen (earth conductor): Tinned annealed copper braiding interwove with polyester yarn.

Sheath: Heavy duty CPE sheath.

»  Dimensions and Weight ..........................................................................................

| Number of Core | Nominal Conductor Area | Strand Size | Insulation Thickness | Core Screen |  | Nominal Sheath Thickness | Nominal Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Strand Size | Area of Screen |  |  |  |
|  | mm² | No/mm | mm | mm | mm² | mm | mm | kg/100m |
| Type 2S 1.1/1kV Collectively Screened |  |  |  |  |  |  |  |  |
| 2 | 1.5 | 30/0.25 | 1.0 | 0.20 | 5.3 | 1.8 | 14.7 | 17 |
| 4 | 1.5 | 30/0.25 | 1.0 | 0.20 | 12.1 | 1.8 | 19.9 | 26 |
| 6 | 1.5 | 30/0.25 | 1.0 | 0.20 | 22.6 | 1.8 | 25.7 | 35 |
| 16 | 1.5 | 30/0.25 | 1.0 | 0.25 | 113.1 | 1.8 | 47.1 | 71 |
| 20 | 1.5 | 30/0.25 | 1.0 | 0.25 | 159.0 | 1.8 | 53.2 | 82 |

48	www.caledonian-cables.co.uk  www.addison-cables.com