Caledonian Mining Cables

AS/NZS 1802:2003 Reeling & Trailing Cables

Type 210 1.1/1.1KV

» Applications .............................................................................................................

These cables are mainly used for hand-held boring machines and drills.

»  Standards

................................................................................................................

Composite Screen

AS/NZS 1802:2003

AS/NZS 1125

AS/NZS 3808

AS/NZS 5000.1

Stranded Tinned Copper Conductor EPR Insulation

Central Extensible Pilot Cradle Separator Insulation Screen

Heavy Duty PCP Sheath

» Construction ............................................................................................................

3×Conductors: Flexible stranded tinned annealed copper conductor.

Insulation: EPR.

Insulation Screen: Semiconductive elastomer.

Composite Screen (earth conductor): Tinned annealed copper braiding interwove with polyester yarn.

Cradle Separator: Semiconductive PCP.

1×Central Extensible Pilot: EPR covered flexible stranded tinned copper conductor.

Sheath: Heavy duty PCP sheath. Heavy duty CPE/CSP sheath can be oﬀered upon request.

»  Dimensions and Weight ..........................................................................................

| Nominal Conductor Area | Strand Size | Insulation Thickness | Core Screen |  | Pilot Conductor |  | Thickness of Sheath | Nominal Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Strand Size | Area of Screen | Strand Size | Thickness of Covering |  |  |  |
| mm² | No/mm | mm | No/mm | mm² | No/mm | mm | mm | mm | kg/100m |
| Type 210 |  |  |  |  |  |  |  |  |  |
| 1.5 | 30/0.25 | 1.4 | 7/0.25 | 5.2 | 24/0.20 | 0.8 | 3.0 | 23.7 | 83 |
| 2.5 | 50/0.25 | 1.5 | 7/0.25 | 5.8 | 24/0.20 | 0.8 | 3.0 | 25.1 | 92 |

www.caledonian-cables.co.uk  www.addison-cables.com	9