Caledonian Mining Cables

Portable Power Cables

Type G Three-Conductor Round Portable Power Cable 2kV

» Applications .............................................................................................................

These cables are designed for use with mobile mining equipment, such as continuous miners, cutting or loading machines, conveyors, drills or pumps�

» Standards .................................................................................................................

ICEA S-75-381/NEMA WC 58 ASTM B 172

ASTM B 33

CAN/CSA C22�2 No� 96

» Construction ............................................................................................................

Stranded Tinned Copper Conductor EPR Insulation

Grounding Conductor Reinforced CPE Jacket

Conductors:

Stranded annealed tinned copper conductor�

Insulation:

Ethylene Propylene Rubber (EPR)�

Grounding Conductor:

Tinned copper conductor with an optional green outer covering�

Jacket:

Reinforced heavy-duty/extra-heavy-duty Chlorinated Polyethylene (CPE), black� (Cables having a nominal outside diameter of more than 2�0 inches require extra-heavy-duty jackets�)

www.caledonian-cables.co.uk  www.addison-cables.com	19

Caledonian Mining Cables

Portable Power Cables

» Options .....................................................................................................................

- Other jacket materials such as CSP/PCP/NBR/PVC are available upon request�

- Two-layer jacket with reinforcing fibre between the two layers can be oﬀered  as an option�

»  Mechanical and Thermal Properties ......................................................................

Minimum Bending Radius: 6×OD

Maximum Conductor Operating Temperature: +90℃

»  Dimensions and Weight

........................................................................................

| Construction | No. of Strands | Grounding Conductor Size | Nominal Insulation Thickness |  | Nominal Jacket Thickness |  | Nominal Overall Diameter |  | Nominal Weight |  | Ampacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of cores×AWG/kcmil | - | AWG/kcmil | inch | mm | inch | mm | inch | mm | lbs/kft | kg/km | A |
| 3×8 | 133 | 10 | 0�06 | 1�5 | 0�125 | 3�2 | 0�91 | 23�1 | 590 | 878 | 59 |
| 3×6 | 168 | 10 | 0�06 | 1�5 | 0�140 | 3�6 | 1�01 | 25�7 | 760 | 1131 | 79 |
| 3×4 | 259 | 8 | 0�06 | 1�5 | 0�155 | 3�9 | 1�17 | 29�7 | 1070 | 1592 | 104 |
| 3×3 | 329 | 8 | 0�06 | 1�5 | 0�155 | 3�9 | 1�24 | 31�5 | 1280 | 1904 | 120 |
| 3×2 | 259 | 8 | 0�06 | 1�5 | 0�155 | 3�9 | 1�34 | 34�0 | 1530 | 2276 | 138 |
| 3×1 | 329 | 7 | 0�08 | 2�0 | 0�170 | 4�3 | 1�51 | 38�4 | 1890 | 2812 | 161 |
| 3×1/0 | 259 | 6 | 0�08 | 2�0 | 0�170 | 4�3 | 1�65 | 41�9 | 2320 | 3452 | 186 |
| 3×2/0 | 329 | 5 | 0�08 | 2�0 | 0�190 | 4�8 | 1�75 | 44�5 | 2700 | 4017 | 215 |
| 3×3/0 | 413 | 4 | 0�08 | 2�0 | 0�190 | 4�8 | 1�89 | 48�0 | 3270 | 4865 | 249 |
| 3×4/0 | 532 | 3 | 0�08 | 2�0 | 0�205 | 5�2 | 2�04 | 51�8 | 3970 | 5907 | 287 |
| 3×250 | 608 | 2 | 0�095 | 2�4 | 0�220 | 5�6 | 2�39 | 60�7 | 5080 | 7558 | 320 |
| 3×300 | 741 | 1 | 0�095 | 2�4 | 0�235 | 6�0 | 2�56 | 65�0 | 6080 | 9046 | 357 |
| 3×350 | 855 | 1 | 0�095 | 2�4 | 0�235 | 6�0 | 2�68 | 68�1 | 7140 | 10623 | 394 |
| 3×400 | 988 | 1/0 | 0�095 | 2�4 | 0�250 | 6�4 | 2�82 | 71�6 | 7780 | 11575 | 430 |
| 3×500 | 1221 | 2/0 | 0�095 | 2�4 | 0�250 | 6�4 | 3�03 | 77�0 | 9065 | 13487 | 487 |

Ampacity-Based on a conductor temperature of 90℃ and an ambient air temperature of 40℃, per ICEA S-75-381�

20	www.caledonian-cables.co.uk  www.addison-cables.com