Caledonian Mining Cables

Portable Power Cables

Type W Two-Conductor Flat Portable Power Cable 2kV

» Applications .............................................................................................................

These flat parallel cables are designed for use on DC mining equipment, such as D�C� shuttle cars, drills, cutting and loading machines�

» Standards .................................................................................................................

ICEA S-75-381/NEMA WC 58 ASTM B 172

ASTM B 33

CAN/CSA C22�2 No� 96

» Construction ............................................................................................................

Conductors:

Stranded annealed tinned copper conductor�

Insulation:

Ethylene Propylene Rubber (EPR)�

Reinforcement: Synthetic yarn� Jacket:

Synthetic Yarn Braid Renforcement Stranded Tinned Copper Conductor EPR Insulation

CPE Jacket

Heavy-duty/extra-heavy-duty Chlorinated Polyethylene (CPE), black� (Cables having a nominal outside diameter of more than 2�0 inches require extra-heavy-duty jackets�)

» Options .....................................................................................................................

- Other jacket materials such as CSP/PCP/NBR/PVC are available upon request�

- Two-layer jacket with reinforcing fibre between the two layers can be oﬀered  as an option�

www.caledonian-cables.co.uk  www.addison-cables.com	3

Caledonian Mining Cables

Portable Power Cables

»  Mechanical and Thermal Properties ......................................................................

Minimum Bending Radius: 6×OD

Maximum Conductor Operating Temperature: +90℃

»  Dimensions and Weight ..........................................................................................

| Construction | No. of Strands | Nominal Insulation Thickness |  | Nominal Jacket Thickness |  | Nominal Overall Diameter Height×Width |  | Nominal Weight |  | Ampacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of cores×AWG/ kcmil | - | inch | mm | inch | mm | inch | mm | lbs/kft | kg/km | A |
| 2×8 | 133 | 0�06 | 1�5 | 0�080 | 2�0 | 0�51×0�84 | 13�0×21�3 | 340 | 506 | 72 |
| 2×6 | 133 | 0�06 | 1�5 | 0�080 | 2�0 | 0�56×0�93 | 14�2×23�6 | 440 | 655 | 95 |
| 2×4 | 259 | 0�06 | 1�5 | 0�095 | 2�4 | 0�61×1�05 | 15�5×26�7 | 550 | 818 | 127 |
| 2×3 | 259 | 0�06 | 1�5 | 0�095 | 2�4 | 0�68×1�14 | 17�3×29�0 | 675 | 1005 | 145 |
| 2×2 | 259 | 0�06 | 1�5 | 0�095 | 2�4 | 0�73×1�24 | 18�5×31�5 | 810 | 1205 | 167 |
| 2×1 | 259 | 0�08 | 2�0 | 0�110 | 2�8 | 0�81×1�40 | 20�6×35�6 | 1020 | 1520 | 191 |
| 2×1/0 | 259 | 0�08 | 2�0 | 0�125 | 3�2 | 0�93×1�51 | 23�6×38�2 | 1265 | 1880 | 217 |
| 2×2/0 | 329 | 0�08 | 2�0 | 0�125 | 3�2 | 0�99×1�63 | 25�1×41�4 | 1515 | 2255 | 250 |
| 2×3/0 | 413 | 0�08 | 2�0 | 0�140 | 3�6 | 1�03×1�77 | 26�2×45�0 | 1810 | 2694 | 286 |
| 2×4/0 | 532 | 0�08 | 2�0 | 0�140 | 3�6 | 1�10×1�89 | 27�9×48�0 | 2175 | 3237 | 328 |

Ampacity-Based on a conductor temperature of 90℃ and an ambient air temperature of 40℃, per ICEA S-75-381�

4	www.caledonian-cables.co.uk  www.addison-cables.com