Caledonian Mining Cables

Portable Welding Cables

Portable Arc-Welding Cable 600V

» Applications .............................................................................................................

These cables are designed for use as ﬂexible welding leads connecting the electrode holder to the welding machine in the secondary circuit of electric arc welding systems�

»  Standards

................................................................................................................

Caledonian Mining Cables

Portable Welding Cables

www.caledonian-cables.co.uk  www.addison-cables.com	57

ICEA S-75-381/NEMA WC 58 ASTM B 172

ASTM B 33

CAN/CSA C22�2 No� 96 UL  1581

» Construction ............................................................................................................

Stranded Annealed Copper Conductor EPR Jacket

Conductors:

Class K/M stranded annealed copper conductor�

Jacket:

Heavy-duty/medium-duty Ethylene Propylene Rubber (EPR)�

» Options .....................................................................................................................

- Other jacket materials such as NR/CSP/PCP/NBR/PVC are available upon request�

- Heavy-duty, two-layer jacket with reinforcement between the two layers can be oﬀered as an option�

www.caledonian-cables.co.uk  www.addison-cables.com	57

Caledonian Mining Cables

Portable Welding Cables

»  Mechanical and Thermal Properties ......................................................................

Minimum Bending Radius: 6×OD

Maximum Conductor Operating Temperature: +90℃

»  Dimensions and Weight

........................................................................................

Caledonian Mining Cables

Portable Welding Cables

58	www.caledonian-cables.co.uk  www.addison-cables.com

| Construction | No. of Strands | Nominal Insulation Thickness |  | Nominal Overall Diameter |  | Nominal Weight |  | Ampacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of cores×AWG/ kcmil | - | inch | mm | inch | mm | lbs/kft | kg/km | A |
| 1×6 | 259 | 0�060 | 1�5 | 0�37 | 9�4 | 124 | 184 | 125 |
| 1×4 | 420 | 0�060 | 1�5 | 0�42 | 10�7 | 180 | 268 | 182 |
| 1×2 | 665 | 0�060 | 1�5 | 0�49 | 12�5 | 268 | 399 | 271 |
| 1×1 | 836 | 0�080 | 2�0 | 0�53 | 13�3 | 319 | 475 | 360 |
| 1×1/0 | 1045 | 0�080 | 2�0 | 0�59 | 14�9 | 415 | 617 | 444 |
| 1×2/0 | 1330 | 0�080 | 2�0 | 0�64 | 16�3 | 508 | 756 | 535 |
| 1×3/0 | 1672 | 0�080 | 2�0 | 0�70 | 17�8 | 628 | 934 | 667 |
| 1×4/0 | 2107 | 0�080 | 2�0 | 0�81 | 20�7 | 775 | 1153 | 809 |
| 1×250 | 2499 | 0�095 | 2�4 | 0�88 | 22�4 | 934 | 1390 | 1048 |
| 1×350 | 3458 | 0�095 | 2�4 | 1�01 | 25�6 | 1267 | 1885 | 1396 |
| 1×500 | 5054 | 0�095 | 2�4 | 1�18 | 30�0 | 1801 | 2680 | 1973 |