Caledonian Mining Cables

Technical Information

Cable Type Definition According to ICEA S-75-381

| Portable cables for use in mining machines, dredges, shovels and similar equipments |  |
| --- | --- |
| Type W | Portable cables without grounding conductors |
| Type G | Portable cables with grounding conductors |
| Type G-GC | Portable cables with grounding conductors and one ground-check conductor |
| Type SHD Flat | Multiconductor portable cables with individually shielded power conductors, and grounding conductors covered with a conducting extrusion layer. |
| Type SHD-PCG | Multiconductor portable cables with individually shielded power conductors, center grounding conductor, and one or more control conductors� |
| Type SHD-CGC | Portable cables with individually shielded power conductors, grounding conductor, and one ground-check conductor in center� |
| Type SHD-GC | Portable cables with individually shielded power conductors, grounding conductor, and one ground-check conductor� |
| Mine power cables for use as connections between units of mine distribution systems |  |
| Type MP-GC | Mine power cables with individually shielded power conductors, grounding conductor, and one ground-check conductor. |

www.caledonian-cables.co.uk  www.addison-cables.com	73