Caledonian Mining Cables

Technical Information

Conductor Identification According to ICEA S-75-381

»  Conductor Identification of Portable Power Cables.............................................

| Power Conductors* |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Two-conductor cables | black | white |  |  |  |
| Three-conductor cables type G, G-GC, SHD-GC, SHD-PCG, SHD-CGC and SHD | black | white | red |  |  |
| Three-conductor cables type W | black | white | green |  |  |
| Four-conductor cables type G | black | white | red | orange |  |
| Four-conductor cables type W | black | white | red | green |  |
| Five-conductor cables type G | black | white | red | orange | blue |
| Five-conductor cables type W | black | white | red | green | orange |
| Control and Ground Check Conductors |  |  |  |  |  |
| Control conductors type PCG | black | white |  |  |  |
| Ground check conductor type G-GC, G-CGC, SHD-GC, SHD-PCG and SHD-CGC | yellow |  |  |  |  |

*: If conducting nonmetallic tapes are used, identification may be by means of stripes or printing in a contrasting colour�

»  Conductor Identification of Mine Power Feeder Cables ......................................

| Power conductors | black | white | red |
| --- | --- | --- | --- |
| Ground check conductors | yellow |  |  |

www.caledonian-cables.co.uk  www.addison-cables.com	77