Caledonian Mining Cables Cables for Underground Mining

NSSHOEU .../3E + ST 0.6/1kV Heavy Duty Flexible Cable

»  Applications

These cables are designed for the connection of mobile equipment and machines under very high mechanical loads in dry and damp areas, outdoors and in explosion hazard areas, particularly in mining and industry, quarries and building sites.

»  Standards

VDE 0250 Part 812

»  Construction

Stranded Tinned Copper Conductor 3GI3 Rubber Insulation

GM1b Rubber Inner Sheath Earth Conductor

Control Cores

5GM5 Rubber Outer Sheath

Conductors: Flexible stranded tinned copper conductor, class 5 according to DIN VDE 0295.

Insulation: Heat resistant EPR type 3GI3.

Earth Conductor: Distributed as spiral of tinned copper wires over core insulating coverings (coding .../3E) or located concentrically between the inner and outer sheaths (coding ...kon).

Control Cores: Laid in the interstices, film wrap.

Inner Sheath: Rubber type GM1b.

Outer Sheath: Chlorinated rubber type 5GM5, abrasion and tear resistant, oil resistant and flame

retardant.

»  Dimensions and Weight

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×2�5+3×2�5/3E | 15�0 | 18�0 | 410 |

www.caledonian-cables.co.uk  www.addison-cables.com	23

Caledonian Mining Cables

Cables for Underground Mining

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×4+3×4/3E | 19�0 | 22�0 | 500 |
| 3×6+3×6/3E | 18�0 | 21�0 | 660 |
| 3×10+3×10/3E | 22�0 | 26�0 | 950 |
| 3×16+3×16/3E | 28�0 | 32�0 | 1350 |
| 3×25+3×16/3E | 29�0 | 33�0 | 1800 |
| 3×50+3×25/3E | 40�0 | 44�0 | 3300 |
| 3×70+3×35/3E | 44�0 | 49�0 | 4360 |
| 3×95+3×50/3E | 52�0 | 57�0 | 5740 |
| 3×120+3×70/3E | 56�0 | 61�0 | 6870 |
| 3×150+3×70/3E | 62�0 | 68�0 | 8140 |
| 3×2�5+3×2�5/3E+3×1�5ST | 18�0 | 20�0 | 500 |
| 3×4+3×4/3E+3×1�5ST | 19�0 | 22�0 | 550 |
| 3×6+3×6/3E+3×1�5ST | 20�0 | 24�0 | 810 |
| 3×10+3×10/3E+3×2�5ST | 24�0 | 28�0 | 1150 |
| 3×16+3×16/3E+3×2�5ST | 28�0 | 32�0 | 1470 |
| 3×25+3×16/3E+3×2�5ST | 30�0 | 34�0 | 1960 |
| 3×35+3×16/3E+3×2�5ST | 34�0 | 38�0 | 2590 |
| 3×50+3×25/3E+3×2�5ST | 41�0 | 46�0 | 3560 |
| 3×70+3×35/3E+3×2�5ST | 44�0 | 49�0 | 4470 |
| 3×95+3×50/3E+3×2�5ST | 52�0 | 57�0 | 5850 |
| 3×120+3×70/3E+3×2�5ST | 51�0 | 56�0 | 6800 |
| 3×150+3×70/3E+3×2�5ST | 59�0 | 64�0 | 8100 |
| 3×2�5/2�5KON | 14�0 | 17�0 | 380 |
| 5×2�5/2�5KON | 18�0 | 21�0 | 560 |
| 5×4/4KON | 20�0 | 24�0 | 710 |
| 5×6/6KON | 20�0 | 24�0 | 910 |
| 10×1�5/1�5KON | 20�0 | 24�0 | 800 |
| 10×2�5/2�5KON | 26�0 | 29�0 | 1100 |

24	www.caledonian-cables.co.uk  www.addison-cables.com