Caledonian Mining Cables Cables for Underground Mining

NTSKCGERLOEU Medium Voltage Coal Cutter Cable

»  Applications

These cables are used for the connection of mobile electrical equipment in underground mines,

e.g. for coal-cutting machines, especially for the use in bretby chains with extreme bending loads under low tensile stress.

»  Standards

VDE 0250 Part 813

»  Construction

3GI3 Rubber Insulation

Stranded Tinned Copper Conductor Earth Conductor

Outer Conductor Layer

Semi Conductive+5GM5 Rubber Inner Sheath Spiral Of Copper And Steel Wires

Pilot Cores

5GM5 Rubber Outer Sheath

Conductors: Flexible stranded tinned copper conductor.

Insulation: Heat resistant 3GI3 rubber based on EPR.

Outer Conductor Layer (for 3.6/6kV): Easy strippable outer conductive layer.

Pilot Cores: Copper/steel conductor capable of expansion and compression with EPR rubber insulation.

Earth Conductor: Spiral of tinned copper wires and a conductive tape.

Inner Sheath: 2 layer design, semi conductive rubber + Rubber type 5GM5.

Armour: Spiral of steel wires, embedded in the outer sheath, fiberglas tape which prevents sheath

exchanging.

Outer Sheath: Rubber type 5GM5, abrasion and tear resistant, oil resistant and flame retardant.

www.caledonian-cables.co.uk  www.addison-cables.com	29

Caledonian Mining Cables

Cables for Underground Mining

»  Dimensions and Weight

1.8/3kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×35+3×(1.5ST+25/3)+UEL | 44�0 | 49�0 | 3900 |
| 3×50+3×(1.5ST+25/3)+UEL | 50�0 | 55�0 | 5100 |
| 3×70+3×(1.5ST+35/3)+UEL | 52�0 | 56�0 | 6200 |
| 3×95+3×(1.5ST+50/3)+UEL | 58�0 | 62�0 | 7500 |
| 3×120+3×(1.5ST+70/3)+UEL | 64�0 | 70�0 | 9350 |

3.6/6kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×35+3×(1.5ST+25/3)+UEL | 49�0 | 54�0 | 5800 |
| 3×50+3×(1.5ST+25/3)+UEL | 56�0 | 61�0 | 6100 |
| 3×70+3×(1.5ST+35/3)+UEL | 57�0 | 62�0 | 6700 |
| 3×95+3×(1.5ST+50/3)+UEL | 62�0 | 66�0 | 8000 |
| 3×120+3×(1.5ST+70/3)+UEL | 67�0 | 71�0 | 10200 |

30	www.caledonian-cables.co.uk  www.addison-cables.com