Caledonian Mining Cables Cables for Underground Mining

NTSCGECWOEU Medium Voltage Coal Cutter Cable

»  Applications

These cables are used for the connection of mobile electrical equipment in underground mines,

e.g. for coal-cutting machines, especially for the use in bretby chains with extreme bending loads under low tensile stress.

»  Standards

VDE 0250 Part 813

»  Construction

System1 System2

Earth Conductor Center Bundle

Semi Conductive+5GM5 Rubber Inner Sheath Spiral Of Steel Wires

5GM5 Rubber Outer Sheath

System1 (1.8/3kV or 3.6/6kV): Flexible stranded tinned copper conductor with heat resistant 3GI3 rubber based on EPR, easy strippable outer conductive layer.

System2 (0.6/1kV): Flexible stranded tinned copper conductor with heat resistant 3GI3 rubber based on EPR.

Earth Conductor: Spiral of tinned copper.

Center Bundle: Control and pilot cores with copper/steel conductors capable of expansion and compression, EPR insulation, optional with fiber optics, covered with tinned copper wires semi conductive rubber sheath.

Inner Sheath: 2 layer design, semi conductive rubber + Rubber type 5GM5.

Armour: Spiral of steel wires, embedded in the outer sheath, fiberglas tape which prevents sheath

exchanging.

Outer Sheath: Rubber type 5GM5, abrasion and tear resistant, oil resistant and flame retardant.

www.caledonian-cables.co.uk  www.addison-cables.com	31

Caledonian Mining Cables

Cables for Underground Mining

»  Dimensions and Weight

1.8/3kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×50+3×(35+35/3)+2×(2×0.75ST)+2×0.75UEL | 62�0 | 66�0 | 7210 |
| 3×70+3×(50+50/3)+2×(2×0.75ST)+2×0.75UEL | 64�0 | 68�0 | 8200 |
| 3×95+3×(70+70/3)+2×(2×0.75ST)+2×0.75UEL | 74�0 | 78�0 | 10300 |

3.6/6kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×35+3×(35+35/3)+2×(2×0.75ST)+2×0.75UEL | 66�0 | 72�0 | 7600 |
| 3×50+3×(50+50/3)+2×(2×0.75ST)+2×0.75UEL | 69�0 | 75�0 | 9200 |
| 3×70+3×(70+70/3)+2×(2×0.75ST)+2×0.75UEL | 76�0 | 81�0 | 11150 |
| 3×95+3×(95+95/3)+2×(2×0.75ST)+2×0.75UEL | 85�0 | 90�0 | 13300 |

32	www.caledonian-cables.co.uk  www.addison-cables.com