Caledonian Mining Cables Cables for Underground Mining

(N)TSCGEWOEU Medium Voltage Trailing Cable

With Anti-Torsion Braid

»  Applications

These cables are used for the connection of electrical equipment, in mines and underground excavations with hazardous environments under particularly high mechanical loads, e.g. high-voltage transformers on power lines in underground mining and tunnelling.

»  Standards

Based on VDE 0250 Part 813

»  Construction

Conductors: Flexible stranded copper

Stranded Copper Conductor 3GI3 Rubber Insulation Earth Conductor

GM1b Inner Sheath Polyester Anti-torsion Braid

5GM5 Rubber Outer Sheath

conductor, class 5 according to DIN VDE 0295.

Inner Conductor Layer: Special rubber compound, conductive.

Insulation: Rubber type 3GI3.

Outer Conductor Layer: Special rubber compound, conductive, easy strippable.

Earth Conductor: Spiral of tinned copper wires.

Inner Sheath: Rubber type GM1b.

Reinforcement: Polyester anti-torsion braid between the jackets embedded.

Outer Sheath: Rubber type 5GM5.

»  Dimensions and Weight

8.7/15kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×150+3×70/3E | 73�0 | 78�0 | 10150 |

www.caledonian-cables.co.uk  www.addison-cables.com	35