Caledonian Mining Cables

Cables for Open-cast Mining

(N)SHOEU 0.6/1kV Flexible cable

»  Applications

These cables are designed for use in open cast mining factories for laying of excavators and conveyor belts with permanent movements between parts of the equipment. These cables are used where the ascending pipe serves as earth conductor, for instance for submersible pumps.

»  Standards

VDE 0250 Part 812

»  Construction

Stranded Copper Conductor EPR Insulation

EPR Inner Sheath 5GM5 Outer Sheath

Conductors: Flexible stranded copper conductor, class 5 according to DIN VDE 0295/IEC 60228.

Insulation: EPR.

Earth Conductor (for –J type): Incorporated as a fourth core or split into three in the outer interstices.

Inner Sheath: EPR.

Outer Sheath: CM type 5GM5.

»  Dimensions and Weight

(N)SHOEU-J

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 3×1�0 | 0�60 | 0�8 | 1�2 | 9�2 | 9�6 | 160 |

40	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables Cables for Open-cast Mining

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 3×1�5 | 0�60 | 0�8 | 1�2 | 10�4 | 12�0 | 180 |
| 3×2�5 | 0�70 | 0�8 | 1�2 | 11�3 | 12�9 | 240 |
| 3×4 | 0�80 | 0�9 | 1�3 | 12�5 | 14�1 | 295 |
| 3×6 | 0�80 | 1�0 | 1�4 | 13�9 | 15�5 | 370 |
| 3×10 | 1�00 | 1�0 | 1�4 | 16�8 | 18�8 | 570 |
| 3x50+3x25/3 | 1�10 | 1�4 | 2�5 | 31�5 | 34�5 | 2500 |
| 3x70+3x35/3 | 1�10 | 1�4 | 2�5 | 36�2 | 39�2 | 3400 |
| 3x95+3x50/3 | 1�20 | 1�7 | 2�8 | 42�0 | 45�0 | 4475 |
| 3x120+3x70/3 | 1�20 | 2�0 | 3�0 | 46�0 | 49�0 | 5400 |
| 4×1�5 | 0�60 | 0�8 | 1�2 | 10�7 | 13�0 | 210 |
| 4×2�5 | 0�70 | 0�9 | 1�4 | 11�7 | 13�7 | 260 |
| 4×4 | 0�80 | 0�9 | 1�4 | 13�7 | 15�7 | 350 |
| 4×6 | 0�80 | 1�0 | 1�4 | 15�2 | 17�2 | 485 |
| 4×10 | 1�00 | 1�0 | 1�6 | 19�0 | 21�0 | 690 |
| 4×16 | 1�00 | 1�2 | 1�8 | 22�0 | 24�0 | 1090 |
| 4×25 | 1�00 | 1�3 | 2�2 | 26�5 | 29�5 | 1600 |
| 4×35 | 1�00 | 1�3 | 2�3 | 30�0 | 33�0 | 2090 |
| 4×50 | 1�10 | 1�5 | 2�6 | 34�5 | 38�5 | 2700 |
| 4×70 | 1�10 | 1�5 | 2�6 | 39�5 | 43�5 | 3620 |
| 4×95 | 1�20 | 1�7 | 2�8 | 45�0 | 49�0 | 4710 |
| 4×120 | 1�20 | 2�0 | 3�3 | 51�0 | 55�0 | 6020 |

(N)SHOEU-O

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 5×1�5 | 0�60 | 0�8 | 1�2 | 12�0 | 14�0 | 240 |
| 5×2�5 | 0�70 | 0�9 | 1�3 | 13�0 | 14�8 | 305 |
| 5×4 | 0�80 | 0�9 | 1�4 | 14�8 | 17�0 | 440 |
| 5×6 | 0�80 | 1�0 | 1�6 | 17�0 | 19�0 | 580 |
| 5×10 | 1�00 | 1�0 | 1�6 | 20�0 | 23�0 | 870 |
| 5×16 | 1�00 | 1�2 | 1�8 | 23�8 | 26�8 | 1280 |
| 5×25 | 1�00 | 1�3 | 2�3 | 29�5 | 32�5 | 1950 |
| 2×1�5 | 0�60 | 0�8 | 1�2 | 9�8 | 11�8 | 155 |
| 7×1�5 | 0�60 | 0�9 | 1�4 | 13�0 | 14�6 | 295 |
| 8×1�5 | 0�60 | 0�9 | 1�4 | 14�0 | 15�6 | 320 |
| 10×1�5 | 0�60 | 1�0 | 1�6 | 15�5 | 17�5 | 390 |

www.caledonian-cables.co.uk  www.addison-cables.com	41