Caledonian Mining Cables Cables for Underground Mining

H07RN8-F 450/750V Harmonized Rubber Cables

»  Applications

These cables are designed for use in submersible pumps and similar applications in water for industrial use up to a water depth of 10 m and water temperature-ranges up to 40℃, also approved for dry, humid and wet applications.

»  Standards

DIN VDE 0282 Part1 and Part 16 HD 22.1

HD 22.16 S1

»  Construction

Stranded Copper Conductor EI4 Insulation

EM2/EM3 Inner Sheath EM2 Outer Sheath

Conductors: Stranded copper conductor, class 5 according to DIN VDE 0295/IEC 60228.

Insulation: Rubber type EI4 according to DIN VDE 0282 Part 16.

Inner Sheath (for ≥ 10 mm² or more than 5 cores): Rubber type EM2/EM3 according to DIN VDE 0282 Part 16.

Outer Sheath: Rubber type EM2 according to DIN VDE 0282 Part 16.

»  Dimensions and Weight

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 1×1�5 | 0�8 | - | 1�4 | 5�7 | 6�7 | 60 |

www.caledonian-cables.co.uk  www.addison-cables.com	9

Caledonian Mining Cables

Cables for Underground Mining

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 2×1�5 | 0�8 | - | 1�5 | 8�5 | 10�5 | 120 |
| 3G1�5 | 0�8 | - | 1�6 | 9�2 | 11�2 | 170 |
| 4G1�5 | 0�8 | - | 1�7 | 10�2 | 12�5 | 210 |
| 5G1�5 | 0�8 | - | 1�8 | 11�2 | 13�5 | 260 |
| 7G1�5 | 0�8 | 1�0 | 1�6 | 14�0 | 17�0 | 360 |
| 12G1�5 | 0�8 | 1�2 | 1�7 | 17�6 | 20�5 | 515 |
| 19G1�5 | 0�8 | 1�4 | 2�1 | 20�7 | 26�3 | 795 |
| 24G1�5 | 0�8 | 1�4 | 2�1 | 24�3 | 28�5 | 920 |
| 1×2�5 | 0�9 | - | 1�4 | 6�3 | 7�5 | 75 |
| 2×2�5 | 0�9 | - | 1�7 | 10�2 | 12�5 | 170 |
| 3G2�5 | 0�9 | - | 1�8 | 10�9 | 13�0 | 230 |
| 4G2�5 | 0�9 | - | 1�9 | 12�1 | 14�5 | 290 |
| 5G2�5 | 0�9 | - | 2�0 | 13�3 | 16�0 | 360 |
| 7G2�5 | 0�9 | 1�1 | 1�7 | 17�0 | 20�0 | 510 |
| 12G2�5 | 0�9 | 1�2 | 1�9 | 20�6 | 23�5 | 740 |
| 19G2�5 | 0�9 | 1�5 | 2�2 | 24�4 | 30�9 | 1190 |
| 24G2�5 | 0�9 | 1�6 | 2�3 | 28�8 | 33�0 | 1525 |
| 1×4 | 1�0 | - | 1�5 | 7�2 | 8�5 | 100 |
| 2×4 | 1�0 | - | 1�8 | 11�8 | 14�5 | 195 |
| 3G4 | 1�0 | - | 1�9 | 12�7 | 15�0 | 305 |
| 4G4 | 1�0 | - | 2�0 | 14�0 | 17�0 | 400 |
| 5G4 | 1�0 | - | 2�2 | 15�6 | 19�0 | 505 |
| 1×6 | 1�0 | - | 1�6 | 7�9 | 9�5 | 130 |
| 2×6 | 1�0 | - | 2�0 | 13�1 | 16�0 | 285 |
| 3G6 | 1�0 | - | 2�1 | 14�1 | 17�0 | 380 |
| 4G6 | 1�0 | - | 2�3 | 15�7 | 19�0 | 550 |
| 5G6 | 1�0 | - | 2�5 | 17�5 | 21�0 | 660 |
| 1×10 | 1�2 | - | 1�8 | 9�5 | 11�5 | 195 |
| 2×10 | 1�2 | 1�2 | 1�9 | 17�7 | 21�5 | 565 |
| 3G10 | 1�2 | 1�3 | 2�0 | 19�1 | 22�5 | 715 |
| 4G10 | 1�2 | 1�4 | 2�0 | 20�9 | 24�5 | 875 |
| 5G10 | 1�2 | 1�4 | 2�2 | 22�9 | 27�0 | 1095 |
| 1×16 | 1�2 | - | 1�9 | 10�8 | 13�0 | 280 |
| 2×16 | 1�2 | 1�3 | 2�0 | 20�2 | 23�5 | 795 |
| 3G16 | 1�2 | 1�4 | 2�1 | 21�8 | 25�5 | 1040 |
| 4G16 | 1�2 | 1�4 | 2�2 | 23�8 | 28�0 | 1280 |
| 5G16 | 1�2 | 1�5 | 2�4 | 26�4 | 31�0 | 1610 |
| 1×25 | 1�4 | - | 2�0 | 12�7 | 15�0 | 405 |
| 4G25 | 1�4 | 1�6 | 2�2 | 28�9 | 33�0 | 1890 |

10	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables Cables for Underground Mining

| Number of Cores×Nominal Cross Section | Insulation Thickness | Thickness of Inner Sheath | Thickness of Outer Sheath | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- | --- | --- | --- |
| No. ×mm² | mm | mm | mm | mm | mm | kg/km |
| 5G25 | 1�4 | 1�7 | 2�7 | 32�0 | 36�0 | 2335 |
| 1×35 | 1�4 | - | 2�2 | 14�3 | 17�0 | 545 |
| 4G35 | 1�4 | 1�7 | 2�7 | 32�5 | 36�5 | 2505 |
| 5G35 | 1�4 | 1�8 | 2�8 | 35�0 | 39�5 | 2718 |
| 1×50 | 1�6 | - | 2�4 | 16�5 | 19�5 | 730 |
| 4G50 | 1�6 | 1�9 | 2�9 | 37�7 | 42�0 | 3350 |
| 5G50 | 1�6 | 2�1 | 3�1 | 41�0 | 46�0 | 3804 |
| 1×70 | 1�6 | - | 2�6 | 18�6 | 22�0 | 955 |
| 4G70 | 1�6 | 2�0 | 3�2 | 42�7 | 47�0 | 4785 |
| 1×95 | 1�8 | - | 2�8 | 20�8 | 24�0 | 1135 |
| 4G95 | 1�8 | 2�3 | 3�6 | 48�4 | 54�0 | 6090 |
| 1×120 | 1�8 | - | 3�0 | 22�8 | 26�5 | 1560 |
| 4G120 | 1�8 | 2�4 | 3�6 | 53�0 | 59�0 | 7550 |
| 5G120 | 1�8 | 2�8 | 4�0 | 59�0 | 65�0 | 8290 |
| 1×150 | 2�0 | - | 3�2 | 25�2 | 29�0 | 1925 |
| 4G150 | 2�0 | 2�6 | 3�9 | 58�0 | 64�0 | 8495 |
| 1×185 | 2�2 | - | 3�4 | 27�6 | 31�5 | 2230 |
| 4G185 | 2�2 | 2�8 | 4�2 | 64�0 | 71�0 | 9850 |
| 1×240 | 2�4 | - | 3�5 | 30�6 | 35�0 | 2945 |
| 1×300 | 2�6 | - | 3�6 | 33�5 | 38�0 | 3495 |
| 1×630 | 3�0 | - | 4�1 | 45�5 | 51�0 | 7020 |

www.caledonian-cables.co.uk  www.addison-cables.com	11