Caledonian Mining Cables Cables for Open-cast Mining

2YSLGCGOEU Signal & Control Cable

»  Applications

These control, signalling and bus cables are used for electric and electronic equipment, such as for measured value and process data processing and automation units in open-cast mining applications, suitable for laying alongside conveyor belts and on material handling equipment.

»  Standards

VDE 0250 Part 812

»  Construction

Stranded Copper Conductor 2YI1 Insulation

EM2 Inner Sheath

Tinned Copper Wire Braid EM2 Outer Sheath

Conductors: Flexible stranded copper conductor, class 5 according to DIN VDE 0295.

Insulation: PE type 2YI1.

Cable Element: Cores are laid-up in pairs in layers with a continuous serving of non-hygroscopic material over the conductor assembly

Inner Sheath: CM type EM2. Screen: Tinned copper wire braid. Outer Sheath: CM type EM2.

»  Dimensions and Weight

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 2×2×1 | 11�0 | 13�0 | 245 |
| 5×2×1 | 16�0 | 19�0 | 440 |
| 10×2×1 | 20�0 | 23�0 | 700 |
| 20×2×1 | 25�0 | 29�0 | 1040 |

www.caledonian-cables.co.uk  www.addison-cables.com	43