Caledonian Mining Cables

Cables for Open-cast Mining

(N)TSCGEWOEU Medium Voltage Reeling Cable

Without Fibre Optics

»  Applications

These cables are used for connection of large mobile equipment such as excavators and spreaders, loading bridges, gantry cranes, construction machines, etc., under very high mechanical loads, in dry or damp environment, also in environments with high explosion risk.

»  Standards

Based on VDE 0250 Part 813

»  Construction

Conductors: Flexible stranded copper

3GI3 Rubber Insulation Stranded Copper Conductor

GM1b/5GM3 Rubber Inner Sheath Polyester Braid

Earth Conductor

5GM3/5GM5 Rubber Outer Sheath

conductor, class 5 according to DIN VDE 0295.

Inner Conductor Layer: Semiconductive layer.

Insulation: EPR type 3GI3.

Outer Conductor Layer: Semiconductive layer.

Earth Conductor: Incorporated as a fourth core or distributed within the outer interstices.

Inner Sheath: Rubber type GM1b/5GM3.

Reinforcement: Polyester anti-torsion braid.

Outer Sheath: Chlorinated rubber type 5GM3/5GM5, flame retardant and oil resistant.

»  Dimensions and Weight

3.6/6kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 39�0 | 42�0 | 2410 |
| 3×35+3×25/3 | 42�0 | 45�0 | 2995 |
| 3×50+3×25/3 | 45�0 | 48�0 | 3645 |
| 3×70+3×35/3 | 50�0 | 54�0 | 4760 |
| 3×95+3×50/3 | 54�0 | 58�0 | 5580 |

44	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables Cables for Open-cast Mining

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×120+3×70/3 | 58�0 | 62�0 | 6690 |
| 3×150+3×70/3 | 63�0 | 67�0 | 7990 |
| 3×185+3×95/3 | 67�0 | 72�0 | 9330 |

6/10 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 40�0 | 43�0 | 2450 |
| 3×35+3×25/3 | 43�0 | 46�0 | 3035 |
| 3×50+3×25/3 | 46�0 | 49�0 | 3690 |
| 3×70+3×35/3 | 51�0 | 55�0 | 4800 |
| 3×95+3×50/3 | 55�0 | 59�0 | 5620 |
| 3×120+3×70/3 | 59�0 | 63�0 | 6740 |
| 3×150+3×70/3 | 64�0 | 68�0 | 8040 |
| 3×185+3×95/3 | 69�0 | 74�0 | 9380 |

8.7/15 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 42�1 | 45�1 | 2707 |
| 3×25+3×50/3 | 43�8 | 46�8 | 3062 |
| 3×35+3×25/3 | 44�9 | 47�9 | 3198 |
| 3×35+3×50/3 | 44�9 | 47�9 | 3382 |
| 3×50+3×25/3 | 49�5 | 53�5 | 4083 |
| 3×50+3×50/3 | 49�5 | 53�5 | 4267 |
| 3×70+3×35/3 | 53�1 | 57�1 | 5028 |
| 3×70+3×50/3 | 53�1 | 57�1 | 5303 |
| 3×95+3×50/3 | 57�3 | 61�3 | 6216 |
| 3×120+3×70/3 | 63�0 | 67�0 | 7673 |
| 3×150+3×70/3 | 66�6 | 70�6 | 8852 |
| 3×185+3×95/3 | 70�5 | 74�5 | 10351 |
| 3×240+3×120/3 | 78�0 | 82�0 | 13125 |
| 3×300+3×150/3 | 84�9 | 89�9 | 16020 |

12/20 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 46�0 | 49�0 | 3050 |
| 3×35+3×25/3 | 49�0 | 52�0 | 3490 |

www.caledonian-cables.co.uk  www.addison-cables.com	45

Caledonian Mining Cables

Cables for Open-cast Mining

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×50+3×25/3 | 53�0 | 57�0 | 4340 |
| 3×70+3×35/3 | 57�0 | 61�0 | 5320 |
| 3×95+3×50/3 | 61�0 | 65�0 | 6360 |
| 3×120+3×70/3 | 67�0 | 71�0 | 7810 |
| 3×150+3×70/3 | 70�0 | 74�0 | 8900 |
| 3×185+3×95/3 | 76�0 | 80�0 | 10700 |

14/25 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 49�9 | 53�9 | 3542 |
| 3×25+3×50/3 | 49�9 | 53�9 | 3726 |
| 3×35+3×25/3 | 52�7 | 56�7 | 4075 |
| 3×35+3×50/3 | 52�7 | 56�7 | 4258 |
| 3×50+3×25/3 | 56�4 | 60�4 | 4872 |
| 3×50+3×50/3 | 56�4 | 60�4 | 5054 |
| 3×70+3×35/3 | 61�5 | 65�5 | 6083 |
| 3×70+3×50/3 | 61�5 | 65�5 | 6356 |
| 3×95+3×50/3 | 65�8 | 69�8 | 7303 |
| 3×120+3×70/3 | 69�9 | 73�9 | 8652 |
| 3×150+3×70/3 | 75�0 | 79�0 | 10139 |
| 3×185+3×95/3 | 78�9 | 82�9 | 11705 |
| 3×240+3×120/3 | 86�2 | 91�2 | 14670 |
| 3×300+3×150/3 | 91�8 | 96�8 | 17332 |

18/30kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 55�0 | 59�0 | 3960 |
| 3×35+3×25/3 | 58�0 | 62�0 | 4550 |
| 3×50+3×25/3 | 63�0 | 67�0 | 5510 |
| 3×70+3×35/3 | 66�0 | 70�0 | 6560 |
| 3×95+3×50/3 | 71�0 | 75�0 | 7850 |
| 3×120+3×70/3 | 76�0 | 80�0 | 9410 |
| 3×150+3×70/3 | 80�0 | 84�0 | 10690 |
| 3×185+3×95/3 | 86�0 | 90�0 | 12550 |

46	www.caledonian-cables.co.uk  www.addison-cables.com