NTSCGEWOEU Medium Voltage Flexible Cable For Use In Water

»  Applications

These cables are designed for operation in water for connection to dredgers, floating docks, pumps, etc., in applications where high mechanical stresses are to be expected, also suitable for use in sewage, salt water and brackish water at water depths of up to 500 m.

»  Standards

VDE 0250 Part 813

»  Construction

Stranded Copper Conductor 3GI3 Rubber Insulation GM1b Rubber Inner Sheath Earth Conductor

5GM3 Rubber Outer Sheath

Conductors: Flexible stranded tinned copper conductor, class 5 according to DIN VDE 0295.

Inner Conductor Layer: Semiconductive layer.

Insulation: Rubber type 3GI3.

Outer Conductor Layer: Semiconductive layer.

Earth Conductor: Split into three in the outer interstices or Individual concentric distributed over core insulating coverings (coding…../3E) or incorporated as a fourth core.

Inner Sheath: Rubber compound type GM1b with characteristics of waterproof and prevention of formation of water bubbles.

Outer Sheath: Waterproof rubber type 5GM3.

»  Dimensions and Weight

1.8/3kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 38�6 | 41�6 | 2480 |
| 3×35+3×25/3 | 42�5 | 45�5 | 3090 |
| 3×50+3×25/3 | 46�0 | 49�0 | 3750 |
| 3×70+3×35/3 | 49�2 | 52�2 | 4620 |
| 3×95+3×50/3 | 57�2 | 61�2 | 6200 |
| 3×120+3×70/3 | 60�9 | 64�9 | 7390 |
| 3×150+3×70/3 | 66�3 | 70�3 | 8830 |
| 3×185+3×95/3 | 69�4 | 73�4 | 10170 |
| 3×25+3×16/3E | 41�3 | 44�3 | 1180 |
| 3×35+3×16/3E | 43�9 | 46�9 | 1650 |
| 3×50+3×25/3E | 47�8 | 50�8 | 2310 |
| 3×70+3×35/3E | 52�5 | 56�5 | 3220 |
| 3×95+3×50/3E | 59�4 | 63�4 | 4335 |
| 3×120+3×70/3E | 65�5 | 69�5 | 5480 |
| 3×150+3×70/3E | 69�2 | 73�2 | 6800 |
| 3×185+3×95/3E | 72�2 | 76�2 | 8375 |

3.6/6kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×16/16 | 47�0 | 52�0 | 3240 |
| 3×25/25 | 50�0 | 55�0 | 3870 |
| 3×35/35 | 55�0 | 55�0 | 4780 |
| 3×50/50 | 59�0 | 64�0 | 5700 |
| 3×70/70 | 64�0 | 70�0 | 7000 |
| 3×25+3×25/3 | 44�7 | 47�7 | 3120 |
| 3×35+3×25/3 | 47�3 | 50�3 | 3600 |
| 3×50+3×25/3 | 52�2 | 56�2 | 4560 |
| 3×70+3×35/3 | 55�5 | 59�5 | 5470 |
| 3×95+3×50/3 | 59�8 | 63�8 | 6570 |
| 3×120+3×70/3 | 65�3 | 69�3 | 8090 |
| 3×150+3×70/3 | 68�9 | 72�9 | 9250 |
| 3×185+3×95/3 | 72�0 | 76�0 | 10600 |
| 3×25+3×16/3E | 46�0 | 49�0 | 3160 |
| 3×35+3×16/3E | 48�6 | 51�6 | 3640 |
| 3×50+3×25/3E | 54�0 | 58�0 | 4600 |
| 3×70+3×35/3E | 57�2 | 61�2 | 5510 |
| 3×95+3×50/3E | 63�8 | 67�8 | 6610 |

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×120+3×70/3E | 68�1 | 72�1 | 8130 |
| 3×150+3×70/3E | 71�8 | 75�8 | 9290 |
| 3×185+3×95/3E | 76�6 | 80�6 | 10840 |

6/10 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×16/16 | 49�0 | 54�0 | 3450 |
| 3×25/25 | 54�0 | 59�0 | 4350 |
| 3×35/35 | 57�0 | 62�0 | 5050 |
| 3×25+3×25/3 | 46�4 | 49�4 | 3320 |
| 3×35+3×25/3 | 49�0 | 52�0 | 3810 |
| 3×50+3×25/3 | 54�0 | 58�0 | 4780 |
| 3×70+3×35/3 | 57�2 | 61�2 | 5700 |
| 3×95+3×50/3 | 61�5 | 65�5 | 6830 |
| 3×120+3×70/3 | 67�0 | 71�0 | 8360 |
| 3×150+3×70/3 | 70�7 | 74�7 | 9530 |
| 3×185+3×95/3 | 73�7 | 77�7 | 10890 |
| 3×25+3×16/3E | 47�8 | 50�8 | 3360 |
| 3×35+3×16/3E | 51�9 | 55�9 | 3850 |
| 3×50+3×25/3E | 55�5 | 59�5 | 4820 |
| 3×70+3×35/3E | 59�0 | 63�0 | 5740 |
| 3×95+3×50/3E | 65�5 | 69�5 | 6870 |
| 3×120+3×70/3E | 69�8 | 73�8 | 8400 |
| 3×150+3×70/3E | 73�5 | 77�5 | 9570 |
| 3×185+3×95/3E | 78�3 | 82�3 | 10930 |

8.7/15 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25+3×25/3 | 52�7 | 56�7 | 4050 |
| 3×35+3×25/3 | 55�3 | 59�3 | 4650 |
| 3×50+3×25/3 | 58�7 | 62�7 | 5390 |
| 3×70+3×35/3 | 63�8 | 67�8 | 6740 |
| 3×95+3×50/3 | 68�1 | 72�1 | 7870 |
| 3×120+3×70/3 | 71�7 | 75�7 | 9150 |
| 3×150+3×70/3 | 77�2 | 81�2 | 10770 |
| 3×185+3×95/3 | 80�2 | 84�2 | 12310 |
| 3×25+3×16/3E | 54�0 | 58�0 | 4090 |
| 3×35+3×16/3E | 56�6 | 60�6 | 4690 |
| 3×50+3×25/3E | 60�3 | 64�3 | 5430 |
| 3×70+3×35/3E | 65�5 | 69�5 | 6780 |

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×95+3×50/3E | 70�3 | 74�3 | 7910 |
| 3×120+3×70/3E | 76�4 | 80�4 | 9190 |
| 3×150+3×70/3E | 80�1 | 84�1 | 10810 |
| 3×185+3×95/3E | 83�1 | 87�1 | 12350 |

12/20 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×25/25 | 66,0 | 72�0 | 6140 |
| 3×35/35 | 69�0 | 75�0 | 6900 |
| 3×25+3×25/3 | 57�0 | 61�0 | 4690 |
| 3×35+3×25/3 | 59�6 | 63�6 | 5260 |
| 3×50+3×25/3 | 64�8 | 68�8 | 6380 |
| 3×70+3×35/3 | 68�1 | 72�1 | 7370 |
| 3×95+3×50/3 | 72�4 | 76�4 | 8600 |
| 3×120+3×70/3 | 77�9 | 81�9 | 10290 |
| 3×150+3×70/3 | 81�5 | 85�5 | 11560 |
| 3×185+3×95/3 | 84�3 | 89�3 | 13000 |
| 3×25+3×16/3E | 58�3 | 62�3 | 4730 |
| 3×35+3×16/3E | 60�9 | 64�9 | 5300 |
| 3×50+3×25/3E | 66�4 | 70�4 | 6420 |
| 3×70+3×35/3E | 69�8 | 73�8 | 7410 |
| 3×95+3×50/3E | 76�4 | 80�4 | 8640 |
| 3×120+3×70/3E | 80�7 | 84�7 | 10330 |
| 3×150+3×70/3E | 84�2 | 89�2 | 11600 |
| 3×185+3×95/3E | 89�0 | 94�0 | 13040 |

14/25 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×50/50 | 81�0 | 88�0 | 9600 |
| 3×70/70 | 86�0 | 93�0 | 11100 |
| 3×95/95 | 92�0 | 99�0 | 13020 |
| 3×25+3×25/3 | 64�4 | 68�4 | 5860 |
| 3×35+3×25/3 | 67�0 | 71�0 | 8390 |
| 3×50+3×25/3 | 70�4 | 74�4 | 7220 |
| 3×70+3×35/3 | 73�7 | 77�7 | 8720 |
| 3×95+3×50/3 | 79�8 | 83�8 | 9950 |
| 3×120+3×70/3 | 83�5 | 87�5 | 11380 |
| 3×150+3×70/3 | 88�7 | 93�7 | 13120 |
| 3×185+3×95/3 | 91�8 | 96�8 | 14770 |
| 3×25+3×16/3E | 65�7 | 69�7 | 5900 |

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×35+3×16/3E | 68�3 | 72�3 | 8430 |
| 3×50+3×25/3E | 72�0 | 76�0 | 7260 |
| 3×70+3×35/3E | 77�2 | 81�2 | 8760 |
| 3×95+3×50/3E | 82�0 | 86�0 | 9990 |
| 3×120+3×70/3E | 87�9 | 92�9 | 11420 |
| 3×150+3×70/3E | 91�6 | 96�6 | 13160 |
| 3×185+3×95/3E | 94�6 | 99�6 | 14810 |

18/30kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×70/70 | 93�0 | 100�0 | 12780 |
| 3×95/95 | 97�0 | 104�0 | 14350 |
| 3×25+3×25/3 | 69�6 | 73�6 | 7010 |
| 3×35+3×25/3 | 72�2 | 76�2 | 7440 |
| 3×50+3×25/3 | 77�4 | 81�4 | 8590 |
| 3×70+3×35/3 | 80�7 | 84�7 | 9670 |
| 3×95+3×50/3 | 84�8 | 89�8 | 11010 |
| 3×120+3×70/3 | 90�2 | 95�2 | 12890 |
| 3×150+3×70/3 | 93�9 | 98�9 | 14260 |
| 3×185+3×95/3 | 96�8 | 102�0 | 15780 |
| 3×25+3×16/3E | 70�9 | 74�9 | 7050 |
| 3×35+3×16/3E | 73�5 | 77�5 | 7490 |
| 3×50+3×25/3E | 79�0 | 83�0 | 8630 |
| 3×70+3×35/3E | 82�4 | 86�4 | 9710 |
| 3×95+3×50/3E | 88�8 | 93�8 | 11050 |
| 3×120+3×70/3E | 93�1 | 98�1 | 12930 |
| 3×150+3×70/3E | 96�8 | 101�8 | 14300 |
| 3×185+3×95/3E | 101�6 | 106�6 | 15820 |

20/35 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×70/70 | 102�0 | 110�0 | 14950 |
| 3×95/95 | 106�0 | 114�0 | 16540 |
| 3×95+3×50/3 | 95�0 | 102�0 | 13670 |
| 3×120+3×70/3 | 101�0 | 109�0 | 15760 |