Caledonian Mining Cables Cables for Open-cast Mining

NTMCGCWOEU Flexible Medium Voltage Single Core Cable

»  Applications

These cables are used for connection in switch-gear and for connection of mobile transformer substations to the overhead line.

»  Standards

VDE 0250 Part 813

»  Construction

Stranded Tinned Copper Conductor 3GI3 Rubber Insulation

Copper Wire Screen

5GM5 Rubber Outer Sheath

Conductors: Flexible stranded tinned copper conductor, class 5 according to DIN VDE 0295.

Inner Conductor Layer: Semiconductive rubber.

Insulation: EPR type 3GI3.

Outer Conductor Layer: Semiconductive rubber.

Screen: Copper wire braid.

Outer Sheath: PCP type 5GM5.

»  Dimensions and Weight

3.6/6kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 1×16/16KON | 19�4 | 21�4 | 601 |
| 1×25/16KON | 21�4 | 23�4 | 825 |
| 1×35/16KON | 22�6 | 24�6 | 882 |

www.caledonian-cables.co.uk  www.addison-cables.com	71

Caledonian Mining Cables

Cables for Open-cast Mining

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 1×50/16KON | 24�6 | 27�6 | 1104 |
| 1×70/16KON | 26�5 | 29�5 | 1346 |
| 1×95/16KON | 28�5 | 31�5 | 1614 |
| 1×120/16KON | 31�2 | 34�2 | 1983 |
| 1×150/25KON | 32�9 | 35�9 | 2300 |
| 1×185/25KON | 34�3 | 37�3 | 2642 |
| 1×240/25KON | 38�6 | 41�6 | 3371 |

6/10 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 1×16/16KON | 20�4 | 22�4 | 644 |
| 1×25/16KON | 22�2 | 24�2 | 791 |
| 1×35/16KON | 23�4 | 25�4 | 1050 |
| 1×50/16KON | 25�4 | 28�4 | 1153 |
| 1×70/16KON | 27�3 | 30�3 | 1399 |
| 1×95/16KON | 29�3 | 32�3 | 1910 |
| 1×120/16KON | 32�0 | 35�0 | 2044 |
| 1×150/25KON | 33�7 | 36�7 | 2364 |
| 1×185/25KON | 35�1 | 38�1 | 2709 |
| 1×240/25KON | 39�4 | 42�4 | 3446 |

8.7/15 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 1×16/16KON | 22�6 | 24�6 | 760 |
| 1×25/16KON | 24�8 | 27�8 | 954 |
| 1×35/16KON | 26�4 | 29�4 | 1101 |
| 1×50/16KON | 28�0 | 31�0 | 1304 |
| 1×70/16KON | 30�5 | 33�5 | 1623 |
| 1×95/16KON | 32�5 | 35�5 | 1912 |
| 1×120/16KON | 34�2 | 37�2 | 2219 |
| 1×150/25KON | 36�9 | 39�9 | 2637 |
| 1×185/25KON | 38�3 | 41�3 | 2995 |
| 1×240/25KON | 41�6 | 44�6 | 3658 |

72	www.caledonian-cables.co.uk  www.addison-cables.com

Caledonian Mining Cables Cables for Open-cast Mining

12/20 kV

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 1×16/16KON | 26�0 | 29�0 | 971 |
| 1×25/16KON | 27�2 | 30�2 | 1090 |
| 1×35/16KON | 28�4 | 31�4 | 1236 |
| 1×50/16KON | 31�0 | 34�0 | 1680 |
| 1×70/16KON | 32�5 | 35�5 | 1776 |
| 1×95/16KON | 34�5 | 37�5 | 2170 |
| 1×120/16KON | 37�2 | 40�2 | 2481 |
| 1×150/25KON | 38�9 | 41�9 | 3020 |
| 1×185/25KON | 40�3 | 43�3 | 3182 |
| 1×240/25KON | 43�6 | 46�6 | 3870 |

www.caledonian-cables.co.uk  www.addison-cables.com	73