Caledonian Mining Cables

Cables for Underground Mining

NSHTOEU 0.6/1kV LHD Cables

»  Applications

These cables are used for frequently changing dynamic loads, such as reeling cables for scoops

(LHDs) in underground mines, suitable for mono-spiral reels and cylindrical reels.

»  Standards

VDE 0250 Part 814

»  Construction

Conductors: Flexible stranded tinned

Stranded Tinned Copper Conductor 3GI3 Insulation

Steel Support Element 5GM5 Inner Sheath Polyester Threads Braid

copper conductor.

Insulation: EPR type 3GI3.

Support Element: Central steel support element.

Inner Sheath: PCP type 5GM5.

5GM5 Outer Sheath

Anti-torsion Braid: Reinforced braid of polyester threads in a vulcanized bond between inner and outer sheath.

Outer Sheath: PCP type 5GM5.

»  Dimensions and Weight

| Number of Cores×Nominal Cross Section | Minimum Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 4x16(6 kN) | 27�5 | 31�5 | 1500 |
| 4x35(12 kN) | 37�5 | 42�0 | 2920 |
| 4x50(12 kN) | 43�0 | 48�0 | 3970 |
| 4x50(30 kN) | 42�0 | 44�0 | 3660 |
| 4x70 (20kN) | 47�0 | 52�0 | 5530 |
| 4x95 (50kN) | 53�0 | 58�0 | 6500 |

14	www.caledonian-cables.co.uk  www.addison-cables.com