Caledonian Mining Cables Cables for Underground Mining

NSSHCGEOEU 0.6/1kV Coal Cutter Cable (High Tensile Stress)

»  Applications

These cables are used for the connection of mobile machines under extremely high mechanical loads, predominantly in mining situations, e.g. for coal-cutting machines, also can be operated via guide pulleys (cable cars) or used as a reeling cable (LHD), suitable for extreme bending loads, high tensile stress, and impact and crushing loads.

»  Standards

VDE 0250 Part 812

»  Construction

Stranded Tinned Copper Conductor Monitoring Conductor

3GI3 Rubber Insulation GM1b Rubber Inner Sheath Copper-Steel Wires Braid Pilot Cores

5GM5 Rubber Outer Sheath

Conductors: Flexible stranded tinned copper conductor.

Insulation: Heat resistant 3GI3 rubber based on EPR.

Outer Conductor Layer: Easy strippable outer conductive layer.

Pilot Cores: Copper and steel conductor capable of expansion and compression with 3GI3 EPR rubber insulation.

Monitoring Conductor: Copper and steel conductor capable of expansion and compression covert with semi conductive rubber compound.

Inner Sheath: Rubber type GM1b.

Armour/Earth Conductor: Concentric earth conductor as tensile-stress-resistant braided armour of combined copper-steel wires.

Outer Sheath: Rubber type 5GM5, abrasion and tear resistant, oil resistant and flame retardant.

www.caledonian-cables.co.uk  www.addison-cables.com	15

Caledonian Mining Cables

Cables for Underground Mining

»  Dimensions and Weight

| Number of Cores×Nominal Cross Section | Minimium Overall Diameter | Maximum Overall Diameter | Nominal Weight |
| --- | --- | --- | --- |
| No. ×mm² | mm | mm | kg/km |
| 3×16/16KON+2ST+UEL | 38�5 | 41�5 | 2430 |
| 3×25/16KON+2ST+UEL | 41�0 | 44�5 | 3050 |
| 3×35/16KON+2ST+UEL | 44�5 | 48�0 | 3620 |
| 3×50/25KON+2ST+UEL | 50�0 | 54�0 | 4810 |
| 3×70/35KON+2ST+UEL | 54�5 | 58�5 | 5890 |
| 3×95/50KON+2ST+UEL | 62�5 | 66�5 | 7800 |
| 3×35/16KON+3ST+3UEL | 44�5 | 48�0 | 3860 |
| 3×50/25KON+3ST+3UEL | 50�0 | 54�0 | 5050 |
| 3×70/35KON+3ST+3UEL | 54�5 | 58�5 | 6000 |
| 3×95/50KON+3ST+3UEL | 62�5 | 66�5 | 8050 |
| 3×120/70KON+3ST+3UEL | 67�5 | 72�0 | 9380 |
| 3×150/70KON+3ST+3UEL | 74�5 | 79�0 | 11120 |

16	www.caledonian-cables.co.uk  www.addison-cables.com